SOUTH WEST DESIGN AFRICA LIMITED (SWDA)

RAEC 18-TANK ATC/NME TANK CLEANING PROJECT — Contract/Tender No. CW-68582 / CW-515913

LIKELY CHEMICALS REGISTER — MSDS SUMMARY & SHOC CARD

Tank Cleaning Operations — Chemicals Identified in HSE Plan Attachment 1 (JHSA)

| RAEC Capability Assessment Reference | Item 5.2(e)-4 / 5.4(d)-2 — "How does the contractor evaluate the health risks presented by use, transport and disposal of chemicals?" |
| --- | --- |
| Requested Evidence | Appropriate SWDA Likely Chemicals MSDSs & SHOC Cards, endorsed by MD |
| Assigned To | Miracle Maduforo (Project Manager — Field Supervisor) |
| Document Date | 06 August 2026 |

This register lists the chemicals/substances likely encountered on SWDA's tank-cleaning campaigns, drawn directly from the hazards already identified in Attachment 1 (JHSA) of the SWDA – RAEC HSE Plan, together with an MSDS summary and a Safe Handling of Chemicals (SHOC) Card for the highest-volume substance.

# 1.0  Likely Chemicals Register — MSDS Summary

| Chemical / Substance | Hazard Class | Key Health/Safety Hazards | Required PPE | Storage / First Aid |
| --- | --- | --- | --- | --- |
| Crude Oil / Gas Oil (Tank Cleaning Diluent) | Flammable Liquid; Irritant; Suspected Carcinogen (long-term exposure) | Skin/eye irritation; inhalation of vapours; flammability near ignition sources | Chemical-resistant gloves, coveralls, safety glasses/goggles; ATEX equipment when handling | Bunded storage away from ignition sources; eye wash if splashed; remove contaminated clothing |
| Detergent (Tank Cleaning Agent) | Irritant | Skin/eye irritation on contact | Gloves, safety glasses, coveralls for routine handling | Store per manufacturer SDS; flush with water if contact occurs |
| Nitrogen (N₂) — Tank Inerting / Purging | Asphyxiant (simple, non-toxic but displaces oxygen) | Asphyxiation risk in confined/poorly-ventilated spaces; no odour warning | 4-Gas Detector (O₂ monitoring), SCBA/Escape Set for CSE work under nitrogen atmosphere | Cylinders/lines secured and leak-checked; evacuate immediately on low-O₂ alarm; move to fresh air |
| Hydrogen Sulfide (H₂S) — Potential Process Residual | Toxic Gas; Asphyxiant | Fatal at high concentration; odour fatigue at low levels | Personal H₂S monitors (mandatory), Breathing Apparatus (SCBA) on alarm | Fixed H₂S monitors/alarms at work area; evacuate on activation; move casualty to fresh air, administer O₂ if trained |

Full manufacturer Safety Data Sheets (SDS) for each substance are held in SWDA's Chemical Inventory file at the Lagos base office and site HSE offices, and are available to Renaissance/RAEC on request.

# 2.0  Sample SHOC (Safe Handling of Chemicals) Card

| Chemical | Crude Oil / Gas Oil — Tank Cleaning Diluent |
| --- | --- |
| Used For | Automatic Tank Cleaning (ATC) dilution/transfer during tank-cleaning operations |
| Physical Form | Liquid — flammable |
| Primary Hazards | Flammable; skin/eye irritant; harmful if inhaled in vapour form; environmental hazard if spilled |
| Required PPE | Chemical-resistant gloves, coveralls, safety glasses, ATEX-rated equipment where sparks/static are possible |
| Handling Precautions | Handle only under Permit to Work; use ATEX pump for transfer; bonded hosing; cordon off works area; no ignition sources nearby; O₂/LEL monitoring where entry is required |
| Storage | Bunded storage area; away from ignition sources and incompatible materials; SDS available at point of storage |
| Spill Response | Use Oil Spill Kit immediately; contain with absorbent material; do not wash into drains; report per Environmental Policy |
| First Aid — Skin Contact | Remove contaminated clothing; wash affected area thoroughly with soap and water |
| First Aid — Eye Contact | Flush immediately with eyewash for at least 15 minutes; seek medical attention |
| First Aid — Inhalation | Move to fresh air immediately; seek medical attention if symptoms persist |
| Emergency Contact | Site HSE Officer; SWDA Emergency Response Contacts (see Section 5.1(g) Safety & Control Systems pack) |

# 3.0  MD Endorsement

I, Gliffeth Wonuigwe, Group CEO / Managing Director of South West Design Africa Limited (SWDA), endorse the Likely Chemicals Register, MSDS Summary and SHOC Card set out above as accurate for SWDA's tank-cleaning operations.

SIGNED: ______________________________

Gliffeth Wonuigwe — Group CEO / Managing Director, SWDA Limited

Date: 06 August 2026