Insert Logo

HAZARD IDENTIFICATION (HAZID) REPORT FOR Utorogu Well 35T

Utorogu Well 35T Acid Stimulation and Nitrogen Lift Project

Client: ND Western / NEPL

Contractor: South West Design Africa (SWDA)

Location: OML 34 – Utorogu Well 35T

Contract Reference: NEPL/OML34AMT/08.06

Date: October 2024

1. PURPOSE

The purpose of this Hazard Identification (HAZID) Study is to systematically identify hazards associated with the execution of the Utorogu Well 35T Acid Stimulation and Nitrogen Lift Project, evaluate the potential consequences and recommend controls required to reduce the risks to As Low As Reasonably Practicable (ALARP).

2. OBJECTIVES

The HAZID aims to:

Identify hazards associated with all project activities.

Identify credible accident scenarios.

Recommend preventive and mitigation measures.

Protect personnel, environment, assets and reputation.

Ensure compliance with ND Western and SWDA HSE requirements.

3. HAZID TEAM

Position Organisation

Operations Representative ND Western

Project Manager SWDA

HSE Manager SWDA

Well Intervention Supervisor SWDA

Coiled Tubing Supervisor SWDA

Pumping Supervisor SWDA

Client Representative ND Western

4. PROJECT ACTIVITIES REVIEWED

The HAZID covered:

Mobilization

Equipment rig up

Crane operations

Pressure testing

Slickline operations

Wellhead integrity verification

Coiled Tubing Operations

Acid stimulation

Nitrogen Lift

Pressure pumping

Chemical handling

Flowback

Waste management

Demobilization

5. HAZID REGISTER

Contract Reference: NEPL/OML34AMT/08.06

| Activity | Hazard | Potential Consequence | Existing Controls | Additional Controls | Risk Owner | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- |
| Mobilization | Road traffic accident | Fatality/injury | Journey Management Plan | Speed & fatigue management | Logistics Coordinator | Medium |
| Equipment Rig-up | Dropped objects | Serious injury | Lifting plan | Barricades and tag lines | Lifting Supervisor | Low |
| Wellhead Testing | High-pressure release | Loss of well control | Pressure testing | Exclusion zone | Well Intervention Supervisor | Medium |
| Slickline | Wire failure | Personnel injury | Certified equipment | Close supervision | Slickline Supervisor | Low |
| Coiled Tubing | Tubing failure | High-pressure release | BOP & CT tests | Continuous monitoring | CT Supervisor | Medium |
| Acid Stimulation | Acid exposure | Chemical burns | PPE & SDS | Emergency shower | Pumping Supervisor | Medium |
| Nitrogen Lift | Nitrogen leak | Asphyxiation | Gas monitoring | Oxygen monitoring | Nitrogen Supervisor | Medium |
| Pressure Pumping | Hose rupture | Serious injury | Inspection | Stand-off distance | Pump Supervisor | Medium |
| Flowback | Hydrocarbon release | Fire/pollution | Gas testing | Fire watch | Flowback Supervisor | Medium |
| Crane Operations | Load swing | Fatality | Certified crane | Wind monitoring | Crane Supervisor | Medium |
| SIMOPS | Activity conflict | Collision | Coordination meetings | Real-time communication | Site Supervisor | Medium |
| Waste Management | Improper disposal | Pollution | Waste segregation | Environmental inspections | HSE Coordinator | Low |

6. MAJOR HAZARDS IDENTIFIED

The HAZID identified the following major hazards:

Loss of Well Control

High Pressure Release

Acid Chemical Exposure

Nitrogen Asphyxiation

Dropped Objects

Crane Failure

Fire and Explosion

Vehicle Accidents

Hydrocarbon Release

Environmental Pollution

Equipment Failure

Simultaneous Operations (SIMOPS)

7. CRITICAL SAFETY CONTROLS

The following critical controls shall remain effective throughout the project:

Permit to Work

Job Safety Analysis

Toolbox Meetings

Certified Personnel

Certified Equipment

Well Control Procedures

Pressure Testing

Gas Monitoring

Emergency Shutdown System

Stop Work Authority

Lifting Plans

Emergency Response Plan

Spill Prevention Measures

PPE Compliance

HSE Supervision

8. ACTION ITEMS

Action Responsibility

Verify all equipment certification SWDA

Confirm emergency contacts ND Western

Conduct pre-job HAZID briefing SWDA

Review Emergency Response Plan Joint

Verify lifting equipment SWDA

Confirm Permit to Work ND Western

Verify competency certificates SWDA

Conduct emergency drill Joint

9. CONCLUSION

The HAZID study concluded that the hazards associated with the Utorogu Well 35T Acid Stimulation and Nitrogen Lift Project can be effectively managed through the implementation of SWDA's HSE Management System, ND Western's Permit to Work System, robust supervision, competent personnel, equipment certification, emergency preparedness and strict compliance with approved procedures.

Implementation of the recommendations identified during this HAZID will reduce project risks to As Low As Reasonably Practicable (ALARP) and support the safe completion of the project.

Assessed by:                                             Signature:				Date:

Approved by:			         Signature:				Date: