Risks Associated with Services and Service-related products

Services

| ID | Activity | Associated Risks | L | I | Risk Ranking | Mitigation and Preventive Control Measures | Action Party | Residual Risks / Notice to Customer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Crane and Forklift Operation. | Dropped Objects resulting in asset damage and major injury/fatality, NPT | 1 | 4 | 4 | Crane and forklift certification and inspection, JHA, Drops Analysis, Training, Rigger and Banks man, PIC, Lifting Plan. | Crane and Forklift Operator and Mates. | 2-Low-Not Significant |
| 2 | Equipment Load-Out | Vehicle break down | 2 | 4 | 8 | Effective PMS, vehicle inspection. | Logistics HOD | 2-Low- Not Significant |
| 3 | Equipment Rig-up | Dropped object resulting in Serious injury and equipment damage | 2 | 4 | 8 | Barricades and tag lines, Lifting plan | Lifting Supervisor | 2-Low- Not Significant |
| 4 | Wellhead Testing | High pressure release resulting in loss of well control | 2 | 4 | 8 | Pressure testing Exclusion zone | Well intervention supervisor | 2-Low-Not significant |
| 5 | Work at height | Fallen object, Fall from height, Equipment damage. | 2 | 5 | 10 | Equipment certification, maintenance and checks, PPE usage, tools tethering, JHA, use of WAH Procedure. | Banksman | 2-Low-Not Significant |
| 6 | Slickline | Wire failure resulting in Personnel injury | 1 | 4 | 4 | Certified equipment, Close supervision | Slickline Supervisor | 2- Low Not Significant |
| 7 | Coiled Tubing | Tubing failure leading to High-pressure release | 2 | 4 | 8 | BOP & CT tests, continuous monitoring | CT Supervisor | 2- Low Not Significant |
| 8 | Acid Stimulation | Acid exposure causing chemical burns | 1 | 5 | 5 | PPE & SDS, Emergency shower | Pumping Supervisor | 2- Low Not Significant |
| 9 | Nitrogen Lift | Nitrogen leak leading to Asphyxiation |  |  |  | Gas monitoring, Oxygen monitoring | Nitrogen Supervisor | 2-- Low Not Significant |
| 10 | Pressure Pumping | Hose rupture causing Serious injury | 2 | 4 | 8 | Inspection, Stand-off distance | Pump Supervisor | 2- Low Not Significant |
| 11 | Flowback | Hydrocarbon release resulting in pollution | 2 | 4 | 8 | Fire Watch, Gas Testing | Flow back Supervisor | 2- Low Not Significant |
| 12 | Community agitations. | Violence, shutdown | 2 | 4 | 8 | Pre engagement with community, obtain signed FSO, presence of RSS, Security assessment, Security operatives’ presence, hired labor from community. | RSS, Rig Manager | 1-Low-Not Significant |
| 13 | Transportation of equipment and personnel to and fro site. | Incident/accident | 1 | 5 | 5 | Vehicle maintenance, inspection and checks, load inspection and proper securing Defensive Driving training, compliance with journey management policy. | Drivers, Journey Manager | 2-Low-Not Significant |
| 14 | Lifting of Cargo | Collapse of suspended load | 1 | 4 | 4 | Responsible person must be present before lifting operation | Contractor | 1-Low-Not significant |
| 15 | Transit between ports | Security | 1 | 5 | 5 | Escort barge with security gun boat | Contractor | 2-Low-Not significant |
| 16 | Berthing Operations | Collision | 1 | 4 | 4 | Caution signs | Contractor | Low- Not Significant |
| 17 | Provision of Security | Theft of secured materials | 1 | 5 | 5 | Adequate lighting, trained security personnel and provision of 24 hours CCTV | Security officers | 2-Low- Not significant |
| 18 | Storage of flammable materials | Fire | 1 | 4 | 4 | Assess materials before storage, install smoke detectors, Provide fire extinguisher at designated points. | Security officers | Low- Not 1-Significant |
| 19 | Electrical operation | Fire | 1 | 4 | 4 | Install smoke detectors, fire extinguishers | Security officers | Low- Not Significant |
| 20 | Supplier performance | Late delivery of critical raw materials | 1 | 4 | 4 | Careful evaluation and selection of suppliers | CEO/HODS | Low- Not Significant |
| 21 | Supplier performance | Failure to supply ordered raw materials | 2 | 4 | 8 | Selection of more than 1 supplier of a product | CEO/ HODs | Low- Not Significant |
| 22 | Supplier performance | Poor supplier evaluation and selection processes | 1 | 4 | 4 | Careful evaluation and selection of suppliers | CEO, HOD’s | Low- Not Significant |
| 23 | Supplier performance | Supply of unsuitable materials, Ineffective purchasing information | 1 | 4 | 4 | Correct description of product information | CEO/HOD’S | Low- Not Significant |
| 24 | Supplier performance | Ineffective purchasing information | 1 | 4 | 4 | Correct description of product information prior to release of PO | CEO/HODs | Low- Not Significant |
| 25 | Availability of competent personnel | Lack or loss of competent personnel | 1 | 3 | 3 | Having available competent replacement personnel to take over the position. | CEO/HRM | Low- Not Significant |
| 26 | Availability of competent personnel | Non-Productive Time (NPT), Incidents | 1 | 3 | 3 | Recruitment as per procedure to ensure availability of competent personnel to man key positions. | CEO/HRM | Low- Not Significant |
| 27 | Lifting and spotting of heavy tubular running gear baskets, Power units | Equipment failure and dropped object | 1 | 5 | 5 | Use of lifting equipment checks, and critical lifting plan | Crane operator and Banksman. | 2 -Low (not Significant) |
| 28 | Lift cylinder operation | Drops at height. | 1 | 5 | 5 | Use of secondary retention | Operator, banksman | 2 -Low (not Significant) |

Service Related-Products

| ID | Service-related Product | Associated Risks | L | I | Risk Ranking | Mitigation and Preventive Control Measures | Action Party | Residual Risks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29 | Air Compressor | Air compressor failure | 4 | 2 | 8 | Spare parts availability, Effective PMS. | Mechanic, Supervisor | Low-Not Significant |
| 30 | Welding and grinding | Arc eye, fire, excessive noise, radiation, vibration. | 2 | 2 | 4 | Machine guards, Screens, fire watch, good housekeeping, rest breaks, appropriate PPE usage, hearing protection, warning and prohibitory signage, Standard Operating Procedures (SOP), Job Hazard Analysis (JHA,),Fire Extinguisher, First aid and firefighting, emergency response team (ERT), Audiometry test | Welders, fitters, QHSE officers | 1 (Not significant) |
| 31 | Sandblasting and painting | Flying particles, fumes, dust, noise, toxicity, respiratory diseases. | 2 | 3 | 6 | 6 Appropriate PPE usage, dampen work area, cordon off work area, use caution signs | HSE officers, sandblaster, painter. | Low-(Not significant) |

Work Environment

| ID | Activity | Associated Risks | L | I | Risk Ranking | Mitigation and Preventive Control Measures | Action Party | Due date | Residual Risks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | Hydrocarbons in the atmosphere | Fire and Explosion | 2 | 5 | 10 | Proper isolation of valves, operators’ competence, HAC compliance. Gas testers, Stop Work Authority, Avoid sources of ignition | Operator | On-going | Low-Not significant |
| 33 | Mixing of chemicals | Chemical spills, burns, corrosion, damage of assets. | 1 | 4 | 4 | Certified and competent operators, tanks inspection, cordon off the hazardous area. Spill boom, eye wash station, first aid, PPE, neutralizers. | Pumping/HWU operators | On-going | Low-Not Significant |
| 34 | High Pressure Lines | Injury | 2 | 4 | 8 | JHA ,Inspection, proper procurement, Warning signs | Supervisors , Operators | On-going | Low-Not significant |
| 35 | Trip and Fall | Injury | 2 | 4 | 8 | Proper Housekeeping, warning signs, | HSE Officers, Operators | On-going | Low-Not significant |
| 36 | Ineffective waste management and disposal |  | 2 | 4 | 8 | Ensure waste management plan is strictly adhered to. Engagement of at least 2 competent community contractors to dispose of waste in a timely manner. | HWO Rig superintendents | On site | Low-Not significant |

L- Likelihood of occurrence

I= Impact of occurrence

Low Risk=1-4       Medium Risk=5-14       High Risk=15-25

Prepared by:                                                                Signature:			           Date:

Signature:			            Date:

Signature:		            	Date:

Reviewed by: 						Signature:			             Date: